10/27/2018:

- Goal

The goal of this project is to use the Tiva C board to collect data from the TSL2591 lux sensor and then push that data wirelessly from a nodeMCU to a thingspeak server as a demonstration of a simple Iot application.

2) Detailed Implimentation

The following is the code I used to both collect and send the data. This code was pushed into the Tiva C board using code composer.

------------------------------------------------------------------------------------

Modified Code:

#include <stdarg.h>

#include <stdbool.h>

#include <stdint.h>

#include "inc/tm4c123gh6pm.h"

#include "inc/hw_i2c.h"

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "inc/hw_gpio.h"

#include "driverlib/i2c.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "driverlib/pin_map.h"

#include "driverlib/uart.h"

#include "driverlib/timer.h"

#include "uartstdio.h"

#include "driverlib/interrupt.h"

#include "driverlib/hibernate.h"

#include "TSL2591_def.h"

#include "ustdlib.h"

void ConfigureUART(void)

//Configures the UART to run at 19200 baud rate

{

SysCtlPeripheralEnable(SYSCTL_PERIPH_UART1); // enables UART module 1

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOB); // enable GPIO port b

GPIOPinConfigure(GPIO_PB0_U1RX); // configures PB0 as RX

GPIOPinConfigure(GPIO_PB1_U1TX); // configures PB1 as TX

GPIOPinTypeUART(GPIO_PORTB_BASE, GPIO_PIN_0 | GPIO_PIN_1); // sets UART pin type

// sets uart clock, boudrate, and which uart to use

UARTConfigSetExpClk(UART1_BASE, SysCtlClockGet(), 115200,

(UART_CONFIG_WLEN_8 | UART_CONFIG_STOP_ONE | UART_CONFIG_PAR_NONE));

}

// prints uart command string

void UARTCommand(unsigned char *str) {

char i=0;

while(str[i] != '\0') {

UARTCharPut(UART1_BASE, str[i]);

i++;

}

}

// prints integer to uart

void UARTInt(int n) {

unsigned char uartChar[32];

int count=0;

while(n > 0) {

uartChar[count] = n%10;

n/=10;

uartChar[count] += '0';

count++;

}

count--;

while(count>=0) {

UARTCharPut(UART1_BASE, uartChar[count]);

count--;

}

}

// calculates the number of chars to print int

int IntLen(int n) {

int count=0;

while(n > 0) {

n/=10;

count++;

}

return count;

}

void I2C0_Init ()

//Configure/initialize the I2C0

{

SysCtlPeripheralEnable (SYSCTL_PERIPH_I2C0); //enables I2C0

SysCtlPeripheralEnable (SYSCTL_PERIPH_GPIOB); //enable PORTB as peripheral

GPIOPinTypeI2C (GPIO_PORTB_BASE, GPIO_PIN_3); //set I2C PB3 as SDA

GPIOPinConfigure (GPIO_PB3_I2C0SDA);

GPIOPinTypeI2CSCL (GPIO_PORTB_BASE, GPIO_PIN_2); //set I2C PB2 as SCLK

GPIOPinConfigure (GPIO_PB2_I2C0SCL);

//Set the clock of the I2C to ensure proper connection

I2CMasterInitExpClk (I2C0_BASE, SysCtlClockGet(), false);

while (I2CMasterBusy (I2C0_BASE)); //wait while the master SDA is busy

}

void I2C0_Write (uint8_t addr, uint8_t N, ...)

//Writes data from master to slave

//Takes the address of the device, the number of arguments, and a variable amount of register addresses to write to

{

//Find the device based on the address given

I2CMasterSlaveAddrSet (I2C0_BASE, addr, false);

while (I2CMasterBusy (I2C0_BASE));

va_list vargs; //variable list to hold the register addresses passed

va_start (vargs, N); //initialize the variable list with the number of arguments

//put the first argument in the list in to the I2C bus

I2CMasterDataPut (I2C0_BASE, va_arg(vargs, uint8_t));

while (I2CMasterBusy (I2C0_BASE));

if (N == 1) //if only 1 argument is passed, send that register command then stop

{

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_SINGLE_SEND);

while (I2CMasterBusy (I2C0_BASE));

va_end (vargs);

}

else

//if more than 1, loop through all the commands until they are all sent

{

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_BURST_SEND_START);

while (I2CMasterBusy (I2C0_BASE));

uint8_t i;

for (i = 1; i < N - 1; i++)

{

//send the next register address to the bus

I2CMasterDataPut (I2C0_BASE, va_arg(vargs, uint8_t));

while (I2CMasterBusy (I2C0_BASE));

//burst send, keeps receiving until the stop signal is received

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_BURST_SEND_CONT);

while (I2CMasterBusy (I2C0_BASE));

}

//puts the last argument on the SDA bus

I2CMasterDataPut (I2C0_BASE, va_arg(vargs, uint8_t));

while (I2CMasterBusy (I2C0_BASE));

//send the finish signal to stop transmission

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_BURST_SEND_FINISH);

while (I2CMasterBusy (I2C0_BASE));

va_end (vargs);

}

}

uint32_t I2C0_Read (uint8_t addr, uint8_t reg)

//Read data from slave to master

//Takes in the address of the device and the register to read from

{

//find the device based on the address given

I2CMasterSlaveAddrSet (I2C0_BASE, addr, false);

while (I2CMasterBusy (I2C0_BASE));

//send the register to be read on to the I2C bus

I2CMasterDataPut (I2C0_BASE, reg);

while (I2CMasterBusy (I2C0_BASE));

//send the send signal to send the register value

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_SINGLE_SEND);

while (I2CMasterBusy (I2C0_BASE));

//set the master to read from the device

I2CMasterSlaveAddrSet (I2C0_BASE, addr, true);

while (I2CMasterBusy (I2C0_BASE));

//send the receive signal to the device

I2CMasterControl (I2C0_BASE, I2C_MASTER_CMD_SINGLE_RECEIVE);

while (I2CMasterBusy (I2C0_BASE));

//return the data read from the bus

return I2CMasterDataGet (I2C0_BASE);

}

void TSL2591_init ()

//Initializes the TSL2591 to have a medium gain,

{

uint32_t x;

x = I2C0_Read (TSL2591_ADDR, (TSL2591_COMMAND_BIT | TSL2591_ID));//read the device ID

if (x == 0x50)

{

//used during debuging to make sure correct ID is received

UARTprintf ("Found Device! %i", x);

}

else

{

UARTprintf ( "Device not Found! %i", x);

while (1){}; //loop here if the dev ID is not correct

}

//configures the TSL2591 to have medium gain adn integration time of 100ms

I2C0_Write (TSL2591_ADDR, 2, (TSL2591_COMMAND_BIT | TSL2591_CONFIG), 0x10);

//enables proper interrupts and power to work with TSL2591

I2C0_Write (TSL2591_ADDR, 2, (TSL2591_COMMAND_BIT | TSL2591_ENABLE),

(TSL2591_ENABLE_POWERON | TSL2591_ENABLE_AEN | TSL2591_ENABLE_AIEN |

TSL2591_ENABLE_NPIEN));

}

void MCU_init ()

//Initializes the MCU

{

UARTCommand("AT+RST\r\n"); // resets the MCU

SysCtlDelay(200000);

UARTCommand("AT+CWMODE=1\r\n"); // sets MCU to station mode

SysCtlDelay(20000);

UARTCommand("AT+CWJAP=\"Zcolo123-2Ghz\",\"DataCenter123#\"\r\n"); // connects MCU to wifi router

SysCtlDelay(200000);

UARTCommand("AT+CIPMUX=0\r\n"); // sets MCU to single connection

SysCtlDelay(20000);

}

void timer0_init ()

//Initializes the timer0 to 30 seconds

{

uint32_t ui32Period;

// enable timer0 and port F

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER0);

TimerConfigure(TIMER0_BASE, TIMER_CFG_PERIODIC);

// calculate timer period for 30 seconds

ui32Period = (SysCtlClockGet() * 30);

TimerLoadSet(TIMER0_BASE, TIMER_A, ui32Period -1);

// enable timer0 interrupt

IntEnable(INT_TIMER0A);

TimerIntEnable(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

IntMasterEnable();

// start timer0

TimerEnable(TIMER0_BASE, TIMER_A);

}

uint32_t GetLuminosity ()

//This function will read the channels of the TSL and returns the calculated value to the caller

{

float atime = 100.0f, again = 25.0f; //the variables to be used to calculate proper lux value

uint16_t ch0, ch1; //variable to hold the channels of the TSL2591

uint32_t cp1, lux1, lux2, lux;

uint32_t x = 1;

x = I2C0_Read (TSL2591_ADDR, (TSL2591_COMMAND_BIT | TSL2591_C0DATAH));

x <<= 16;

x |= I2C0_Read (TSL2591_ADDR, (TSL2591_COMMAND_BIT | TSL2591_C0DATAL));

ch1 = x>>16;

ch0 = x & 0xFFFF;

cp1 = (uint32_t) (atime * again) / TSL2591_LUX_DF;

lux1 = (uint32_t) ((float) ch0 - (TSL2591_LUX_COEFB * (float) ch1)) / cp1;

lux2 = (uint32_t) ((TSL2591_LUX_COEFC * (float) ch0) - (TSL2591_LUX_COEFD * (float) ch1)) / cp1;

lux = (lux1 > lux2) ? lux1: lux2;

return lux;

}

void main (void)

{

//set the main clock to run at 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

ConfigureUART (); //configure the UART of Tiva C

I2C0_Init (); //initialize the I2C0 of Tiva C

TSL2591_init (); //initialize the TSL2591

MCU_init(); //initialize the nodeMCU

timer0_init(); //initialize the timer0

while (1)

{};

}

void Timer0IntHandler(void)

{

// Clear the timer interrupt

TimerIntClear(TIMER0_BASE, TIMER_TIMA_TIMEOUT);

uint32_t lux = 0, i;

uint32_t luxAvg = 0;

for (i = 0; i < 20; i++)

//finds the average of the lux channel to send through uart

{

lux = GetLuminosity ();

luxAvg += lux;

}

luxAvg = luxAvg/20;

// connect to thingspeak using TCP connection

UARTCommand("AT+CIPSTART=\"TCP\",\"184.106.153.149\",80\r\n");

SysCtlDelay(200000);

// calculate and send how many characters are necessary

UARTCommand("AT+CIPSEND=");

SysCtlDelay(20);

UARTInt(72+IntLen(luxAvg));

SysCtlDelay(20);

UARTCommand("\r\n");

SysCtlDelay(20000);

// send data to thingspeak

UARTCommand("GET https://api.thingspeak.com/update?api_key=0XLLUY55C4GFKPH7&field1=");

SysCtlDelay(20);

UARTInt(luxAvg);

SysCtlDelay(20);

UARTCommand("\r\n");

SysCtlDelay(20000);

// blink LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

SysCtlDelay(4000000);

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 0);

}

------------------------------------------------------------------------------------

3) Schematics

	The following is a schematic of the connections needed to use these electronics together and perform the necessary operations.



	Hand drawn schematic for the project

4) Video Links

Video Link: https://www.youtube.com/watch?v=tHmhrQhCHYM

5) Screenshots

Here is some data it collecting for a couple hours in my house while I was shining lights on it and casting shadows on it.

Chart for data while in my house.

Chart with approximately 18 hours of data while on a window sill

The first part of the last chart was from indoor light, but when it drops down late at night on november 4 is when it was put on the window. There is also a minor error in the code which causes it to not send data if the reading is zero, which is why there are large gaps especially late at night.

6) Conclusions

Overall, this project was far more difficult than I had anticipated. Getting a few different devices to talk to each other especially using different communication interfaces was a challenge. In the end it seems I pulled through and got it working. Of all the work I have done in this class so far, this seems to be, by far, the most useful application. Sensors are always needed to gather more data, and transmitting it to a website is a pretty realistic end goal for a piece of technology like this.